Ship’s particulars

LPG/C   JEANNE-MARIE

SUMMARY

1		VESSEL’S  CARACTERISTICS

2		CARGO TANKS CARACTERISTICS

3		CARGO HANDLING EQUIPMENT

4		CARGO OPERATIONS

1	VESSEL’S CARACTERISTICS

1.1	CARACTERISTICS

Name		:	JEANNE-MARIE

Motor ship	:	78 679 m3  	0.250/0.400 barg  IMO * 0.250 barg USCG

* - 48 °C.

Owner	:	GAZ NORMANDIE c/o GEOGAS MARITIME

10 rue Pergolèse 75116 Paris France

Flag	:FRENCH ANTARCTIC TERRITORY	Official number :	FK 924633

Keel laid	:	3 May 2004

Delivered	:	11 January 2005

Manager	:	Marima c/o V.SHIPS FRANCE (tankers)

75116	PARIS    FRANCE

IMO   Nr	:	9290270

1.2	TONNAGE

London  1969		GRT	=	 46632 UMS

NRT	=	15 461 UMS

Suez			GRT	=	48763,42 tx

NRT	=	41655,16 tx

Panama		GRT	=	Not applicable

NRT	=	Not applicable

1.3	HULL

L.o.a.			:	224.5 m	(736.54 Ft)

L.b.p.			:	213.0 m	(698.81 Ft)

Breadth		:	36.0 m		(118.10 Ft)

Depth			:	22.30 m	(73.16 Ft)

Dist. keel / antenna	:	50.08 m / 48.63 m tilting

Parallel body length	:	103.8 m loaded	83.4 m on ballast

Lightship displacement:	17 562.7 mt

1.4	DRAUGHT

With full cargo and full bunkers .

Propane 	11.09 m		Corresponding DWT 	= 49 323.8 mt

Butane		11.35 m		Corresponding DWT 	= 51 047.3 mt

Summer Draft	11.70 m		Summer Deadweight	= 53 208  mt

Immersion in S.W.

- at summer freeboard                      :  t.p.c. = 67.2 t/cm    mtc : 920.3 tm.

- at normal ballast draught 7,31m    :  t.p.c. = 62.7 t/cm    mtc : 770.3 tm

-fresh water allowance: 0.235 m

1.5	CLASS

## Bureau Veritas 	N° 04511F

## I +HULL+ MACH

Liquefied Gas Carrier/LPG

Unrestricted navigation

+VeriSTAR-HULL

+ AUT-UMS

SYS-NEQ-1

MON-SHAFT

INWATERSURVEY

Vessel built according to applicable IMO Conventions, IGC Code, USCG CFR, MTSL Japan, NKKK tank calibration, OCIMF recommendations and guidelines.

1.7	MACHINERY

1.7.1	Main engine

Type & made	:         	1 set of diesel engine SULZER 7RTA62U-B

Power		:	Design values :

Maximum continuous rating  = 21770 hp at 115.0 rpm

Normal continuous rating (90%)  = 19590 hp at 111.0 rpm

Shaft line 	:	No spare shaft/propeller on board.

## Fuel used	:	HFO up to 600 Cst at 50°C

Normal service:	17.3 knots (design draft at NCR with 15% sea margin)

1.7.2	Auxiliaries

Type & maker	:3 sets of 4-stroke STX engine 1260 kW at max continuous output                               720 rpm. Alternator output 1208 KW.

## Grade of fuel	:	HFO up to 600 Cst at 50°C

Emergency Generator :1 STX engine set 250 kW at 1 800 rpm, alternator output 180 KW Grade of fuel MDO

1.7.3	Bow thruster	:	Not fitted

2	CARGO  TANKS  CARACTERISTICS

2.1	Volume at 100%:	Tank 1 P	:	8 394.5 m3

(20°C)			Tank 1	S	:	8 388.3 m3

Approx

Tank 2	P	:	10 475.3 m3

Tank 2 S	:	10 468.6 m3

Tank 3 P 	:	10 512.5 m3

Tank 3 S	:	10 504.4 m3

Tank 4 P	:	9 938.1 m3

Tank 4 S	: 	9 932.5 m3

Total 100%	:	78 614.2 m3

2.2	TRANSPORTABLE PRODUCTS

As per Certificate of Fitness.

BUTANE

PROPANE

BUTANE / PROPANE MIXTURE

3	CARGO HANDLING EQUIPMENT

3.1	CARGO  PUMPS

Number	:	8  (2 per tank)

## Maker		:	Hamworthy KSE Savenaoj A/S

Type		:	Vertical centrifugal deepwell pumps

Model DW 250/200-3-K+I

Capacity	:	550 m3/h, head 120 MLC, maxi S.G. = 0.61

3.2	BOOSTER PUMPS

Number	:	2  (able to work in parallel or in series)

Maker		:	Hamworthy KSE Savenaoj A/S

Model NMB 150E

Type		:	Horizontal centrifugal

Capacity	:	550 m3/h, head 120 mLC, maxi S.G. = 0.61

3.3	CARGO COMPRESSORS

Number			:	4

Located			:	Reliquefaction plant room (upper deck)

Make				:	Sulzer 3K 140-3A, 248 KW

Type				:	Vertical, 3 cylinders, 3 stages oil free reciprocating

Frigorific production		:	For commercial Propane (Ethane 4%), sea

water temp. 32°C, suction temp. –20°C :

Capacity : 222 kW  (361 Kcal)

One unit is able to keep the temperature/pressure of two tanks Propane with outside IMO temperature conditions.

3.8	CRANES

Handling		:	Cargo hose	    Provision	         Provision

Number		:	     1 set	       1 set	             1 set

Located		:	Manifolds	       Port Acc.	        Starboard Acc.

Capacity		:	5.0 tonnes             5.0 tonnes            	5.0 tonnes

Cargo hose crane outreach	: 	1 m from ship side at farest manifold,

6 m from ship side at position of crane

4	CARGO OPERATIONS

V

BALLAST

LOAD

LOA 224.5 m

LPP 213 m

111.9 m

112.6 m

68.9 m

48.6 m

34.8 m

34.9 m

###### JEANNE-MARIE

# G E O G A S